# IR Temperature Sensing Project

6/25/19 - 6/26/19

Project Objective

Project Key Results/Deliverables

Project Timeline

Rationale

Data Collection Protocol

Example Scenario Data

Noise in IR Temperature Sensing

#1 while standing

Sitting Only

#1 while sitting (male)

#2 Only

#2, then #1

Preliminary Conclusions

Detection of “use”

Characterization of #1 vs. #2

Next Steps

# Project Objective

- Demonstrate ability automatically classify toilet-use-case-scenarios for passive user experience.

# Project Key Results/Deliverables

- Initial assessment of data, sensors, and feasibility of signal processing route.

- Correctly classify if user has used a toilet with probability score > 90%.

- Correctly classify new incoming data as #1 or #2 with probability score > 80%.

- Written report of methodology and results, alongside assessment of future feasibility to reach future probably score of > 99%.

# Project Timeline

- Approximately 80 hours, ending September 30, 2019.

# Rationale

- Contact-less IR sensor can detect temperature from a distance

  - Attaches easily to toilet-bowl clip and pointed slightly towards inside

  - Everything exiting a human will be at body temperature and thus detectable

- Contact-less IR/photodiode sensor can detect IR RGB color wavelength intensity

  - Water vs #1 vs #2 have distinct color differences that may be identifiable

# Data Collection Protocol

- Note: Brian can assist in the collection of more example/training data using the setup below.

- Note: Brian can provide existing raw data.

- Arduino sketch loops through IR temperature reading to Serial out every 250 microseconds.

  - Run, plot in Google Sheets

- Taped IR sensor to a clip so I could easily adjust angle for testing

  - Most tests carried out at “angle 1” (left)

# Example Scenario Data

## Noise in IR Temperature Sensing

- Baseline readings in position with no use.

## #1 while standing

- Gradual rise in temperature from baseline

- Stays elevated, but gradually goes down

- Flushing quickly brings temperature down. Returns almost to baseline, not quite.

## Sitting Only

- Sitting down increases temperature slightly

## #1 while sitting (male)

- Same gradual rise in temperature above “sitting” baseline.

## #2 Only

- Not the same kind of gradual rise or general temperature behavior as #1.

- Often see “signature” spikes (depends on “solidity” of #2?)

## #2, then #1

- See separate “phases,” with resembling characteristics of #2, and #1 in different parts of the plot.

# Preliminary Conclusions

## Detection of “use”

- Appears to easily identify if someone uses the toilet

  - Very clear delineation between baseline noise and use

  - Clear delineation between sitting (only) and sitting and use

## Characterization of #1 vs. #2

- There are temperature “signatures” that appear to be able to differentiate between #1 and #2.

## Next Steps for IR Temp

- More data collection will help

  - More “use cases”

    - Female

    - Various “kinds” of #2’s

  - Test in really hot environment

    - Room temperature close to body temperature

      - Still see detection?

## RGB Sensor

- Data enclosed in Excel file.

- RBG sensor on right in image below (IR Temp on left)